The Indoor Planting System architecture has been carefully designed to integrate IoT, machine learning, voice assistance, and mobile interfaces to help visually challenged individuals monitor and take care of plants on their own. The system consists of hardware and software components spread over several platforms and services, collaborating in real-time to enable accessibility, automation, and smart analysis.

Hardware (Edge) Layer – ESP32-Based Smart Monitoring

At the center of the system is the ESP32 micro-controller, which serves as the main sensing and actuating center. It is integrated with a range of sensors and devices such as:

- Soil Moisture Sensor – measures soil water content.

- LDR Sensor - measure amount of sunlight the plant is getting

- Water Level Sensor – installed in the water tank to avoid water overflow when irrigating.

- Temperature and Humidity Sensor – monitors conditions in the environment.

- Two IR Sensors:

- For detecting intruders.

- For the measurement of plant height.

ESP32-CAM – takes snaps of the plant at regular intervals.

The ESP32 transmits real-time sensor data to the Blynk Cloud over Wi-Fi, as well as uploads images taken using a Flask API for post-processing.

Cloud Backend and Integration

Two core servers run the system:

1. Node.js/Express.js Server

Serves as a middleware to retrieve data from Blynk Cloud. Handles multiple HTTP APIs that retrieve sensor and motor data. Provides the data to the Flutter mobile app. Supports RESTful communication to enhance scalability and frontend integration.

2. Flask Server

Receives images from ESP32-CAM. Uses Roboflow's cloud-hosted ML model API for:

- Plant Disease Detection. To detect plant disease

- Fruit Detection. To detect the pre

- Voice Chatbot Interface, in which the users can question by speaking.Google Gemini LLM powers the chatbot and replies accordingly on the basis of data from the 	plant as well as common horticultural information.

- An Audio-based SOS Model that responds to emergency noises such as cries or alarms and informs the user.

These services make the system intelligent, responsive, and secure.

Flutter Mobile App – Accessibility Frontend

The mobile app is developed with Flutter and is the main user interface. It provides:

- Accessible UI designed specifically for the visually impaired, which includes:

- Voice-over descriptions of data.

- Vibration feedback on major interactions.

- Real-time plant monitoring via  status and danger alerts.

- Thumb zone Voice Assistant Button for fast auditory overviews.

- Danger Alert Page that rings an alarm and asks to acknowledge when critical thresholds (such as fire, intrusion) are exceeded.

- Flask and Node API integration to retrieve and show ML predictions, sensor readings, and chatbot answers.

Machine Learning & Voice AI Integration

Two ML-based services are of crucial importance:

- Roboflow Detection API: Identifies plant health and fruit availability from camera photos.

- Gemini LLM + Flask-based Chatbot: Recognizes voice questions and offers detailed plant-care answers.

Both systems execute inference remotely to maintain the app light and snappy.

Security & Performance

- Secure API exchanges between frontend and backend with tokens and validation.

- Optimized image uploading and caching for seamless ML inference.

- Error-handling to notify users on device disconnection or sensor malfunctions.

This design not only supports autonomy among visually impaired individuals but also shows an effective merging of IoT, AI, and mobile technologies into one cohesive smart agriculture solution.