- System Overview:

This system automates plant care and provides detailed plant health information. It integrates an ESP32 microcontroller with sensors, actuators, and a machine learning (ML) model. The mobile app (using Blynk) is designed for visually challenged users, featuring voice assistance and haptic feedback for accessibility.

- Architecture Overview

3. System Components

Hardware

1. Processor: ESP32 (main controller) and ESP32-CAM (for ML tasks).

2. Sensors:

- Soil Moisture Sensor: Monitors soil hydration.

- LDR (Light Sensor): Measures light intensity.

- Temperature & Humidity Sensor (DHT22): Records environmental conditions.

- Water Level Sensor: Monitors water tank levels.

- pH Sensor: Measures soil pH for nutrient assessment.

3. Actuator:

- DC Motor: Pumps water based on soil moisture and water level conditions.

4. Camera:

- ESP32-CAM: Captures images for health and growth monitoring.

5. Power Supply:

- Ensure sufficient power (5V for ESP32 and peripherals).

Software

1. Microcontroller Logic (ESP32):

- Data acquisition from sensors.

- Decision-making for watering and motor control.

- Communication with the Blynk app.

- Scheduled activation of ESP32-CAM for ML-based plant health checks.

2. Machine Learning (ML) Model:

- Model Type: Lightweight CNN (e.g., MobileNet) optimized with TensorFlow Lite.

- Functionality:

- Detect plant health (healthy, decayed, or dead).

- Analyze growth using color and size differences.

- Deployment: Use ESP32-CAM with TensorFlow Lite Micro.

3. Mobile App (Blynk):

- Displays sensor data (e.g., soil moisture, light intensity, pH, water level).

- Notifications for visually challenged users:

- Voice Assistance: Audio Messages to update the plant information to user.

- Haptic Feedback: Vibrations for Navigation.

4. Working Principle

Sensor Integration and Decision-Making:

1. Soil Moisture:

- If moisture drops below a set threshold, the DC motor pumps water.

- Stops when soil moisture is sufficient or the water level is low.

2. Water Level:

- Prevents water pump activation if the tank is empty.

3. Light Sensor:

- Tracks light exposure for plant health.

4. pH Sensor:

- Sends alerts if pH levels are unsuitable.

5. Temperature & Humidity Sensor:

- Sends alerts of temperature and Humidity of the plat environment .

Machine Learning Workflow:

1. Image Capture:

- ESP32-CAM captures plant images every 3–4 days.

2. Model Inference:

- Classifies plant health (healthy, decayed, or dead).

- Measures growth based on color and size.

3. Action:

- Sends plant health and growth reports to the app.

Blynk App:

Accessible Features:

Custom voice alerts for each metric.

Large, high-contrast buttons and simplified UI.

Widgets for:

Real-time data Monitoring.

Manual motor control (if needed).